Some patients may have difficulty tolerating CPAP for many different reasons which are discussed below:

- Lack of Humidification: In general, CPAP tends to dry out the air, therefore, extra humidification is usually required. A pass-over humidifier and a heated humidifier are available to help add moisture or humidity to the air moving through the breathing circuit. A heated humidifier produces more moisture than the pass-over humidifier. Occasionally, too much humidification may be produced causing irritation of the nasal passages and moisture to collect in the tubing and mask. Lowering the number on the heated humidifier usually remedies this problem.

- Temperature: The air coming from the CPAP/BiPAP can be cold. This can be corrected by ensuring the room temperature is warmer or by using a heated humidifier.

- Nasal Allergies: Sometimes nasal allergies can be exacerbated by CPAP therapy. Any known allergens in the room should be removed. Frequent vacuuming may be required. Raise the CPAP blower to a higher level if it is located on the floor. Elimination of animals from the bedroom may be helpful. Ensure that CPAP filters are changed frequently. Use only distilled water in the humidifier and replace it daily. Try the humidifier with and without heat to determine which is best for you. An oral antihistamine may be taken before sleep if needed.

- Nasal Dryness: Use a humidifier, either heated or regular pass-over. Use a nasal saline spray before sleep at night.

- Nasal Congestion: Check for nasal allergies and take appropriate action to treat them (see step #3). nasal dilator during sleep at night may helpful, however, you should check with your doctor first.

- Nasal Irritation: Use a humidifier, preferably a heated one. Use a saline spray before sleep. Consider using a small amount of a non-petroleum-based cream in your nostrils.

- Mask Pressure: The mask should not be too tight on the face. If it needs to be very tight for adequate seal, try a different type of mask—there are many types from which to choose. Consider nasal pillows to reduce mask contact with the skin.

- Skin Allergies: Make sure you have a hypoallergenic mask made of silicone. If experience a reaction to the mask cushion, consider a nasal mask with pillows to reduce mask contact with the skin.

- Mouth Opening: Any of the above recommendations may be helpful for your dry mouth. To ensure CPAP pressure is optimal, you may need to get a full-face mask.

- Falling Asleep: If have trouble getting to sleep, try getting out of bed and doing something else. Preferably move to another room and return to bed only when sleepy. Establish a routine for an hour or so each night before bedtime, such as, reading, taking a warm bath, light exercise, such as stretching, or just relaxing quietly. Avoid too much mental stimulation during the hour prior to bedtime.

- Stress: Almost everyone experiences an occasional night of lost or disturbed sleep. This can be caused by stress. Try the above suggestions for falling asleep.

- Naps: If you are having trouble falling asleep at night, avoid naps in the early afternoon or evening.

- Getting Up: No matter how poorly you have slept, always set your alarm to arise at the same time each morning.

- Exercise: Regular exercise can be an effective aid to sleep. It releases energy and eases mental tensions. Do not exercise strenuously just before bedtime.

- Noises: Occasional loud noises from aircraft, streets, or highways disturb sleep even in people who do not awaken and who do not remember the noise in the morning. These sleep disturbances can reduce restful sleep. People who sleep near excessive noise should try heavy curtains in their bedrooms or ear plugs to help block the outside noise.

- Hunger: A light snack or warm milk seems to help some people get to sleep. Always avoid coffee, tea, and cola near bedtime. Also, void heavy meals 2 hours before bedtime.

- Sleeping Patterns: Everyone has a unique sleeping pattern. Some adults need 10 hours of sleep a night—others need only 6. Many people function best at 8 hours of sleep per night. Your requirement for sleep is unique, and what is effective for others may not be effective for you. Find your individual sleep need, and maintain a consistent schedule for sleeping.

- Symptom of Medical Problems: Sleeping problems may signal a medical condition such as anxiety, depression, or other disorder. It is important to get a proper diagnosis and treatment of the underlying cause of a chronic sleep disorder.

- Pregnancy: Excessive sleepiness the first 3 months of pregnancy is normal. Pregnant women also tend to sleep about 2 additional hours a night.

- Medications/Alcohol: An occasional sleeping pill may be of some benefit, but chronic (nightly) use of sleeping pills may actually hinder good sleep. Sleeping medications should be used with caution, for the short-term management of a sleep complaint, and only upon the advice of a physician. Do not increase the dosage yourself. If you feel that the medication is losing its effect, report this to your doctor. Although alcohol may help to induce sleep, the chronic use of larger quantities of alcohol causes disturbed sleep and dependency.

- Power outage: Remove mask and try to go back to sleep. During and electrical storm, using the CPAP equipment is not a problem. CPAP equipment is not a conductor of electricity. In addition, the mask has been designed to avoid suffocation if the patient is breathing with the mask while the power is off. Although it may not be comfortable, it should not cause you any harm.